Final Project Thermostat Reflection

For this final project, I designed and implemented a fully functional thermostat system using a Raspberry Pi. The system integrates multiple hardware peripherals including GPIO devices, an I2C temperature sensor, an LCD display, and UART serial communication. The thermostat operates using a state machine model that controls three operating states: OFF, HEAT, and COOL. Throughout this project, I gained practical experience integrating hardware and software while applying embedded systems concepts learned in this course.

Peripheral Integration

This project uses several hardware peripherals. First, General Purpose Input/Output (GPIO) was used extensively. GPIO pins were used to control two LEDs, detect three button presses, and interface with a 16x2 LCD display. The red and blue LEDs were controlled using PWM (Pulse Width Modulation) to create fading effects when the system was actively heating or cooling. The buttons were configured using interrupts through the gpiozero library, allowing the program to detect button presses without continuously polling the pins. This interrupt-driven design improves efficiency and responsiveness.

The thermostat uses an I2C peripheral to communicate with the AHTx0 temperature sensor. The I2C interface was initialized using the board.I2C() method, and the Adafruit AHTx0 library was used to retrieve temperature readings. The sensor returns temperature in Celsius, which was converted to Fahrenheit inside the code. This demonstrates proper use of peripheral abstraction libraries to simplify hardware communication.

The LCD display was connected through digital GPIO lines using the digitalio and adafruit_character_lcd libraries. The display shows the current date and time on the first line and alternates the second line between the current temperature and the thermostat’s state with its setpoint. This alternating display approach allows multiple pieces of information to be shown on a limited 16x2 screen.

Finally, UART serial communication was implemented to send status updates to a TemperatureServer every 30 seconds. The serial port was configured with a baud rate of 115200 and outputs a comma-delimited string containing the current state, temperature, and setpoint. This demonstrates use of the Raspberry Pi’s hardware serial interface.

State Machine Design

The thermostat operates using a formal state machine model. The three states are OFF, HEAT, and COOL. A single button cycles between these states. The state machine ensures clean transitions and predictable behavior.

In the OFF state, both LEDs remain off regardless of temperature.
In the HEAT state:

- If the current temperature is below the setpoint, the red LED fades in and out to indicate active heating.

- If the temperature is above the setpoint, the red LED remains solid.

In the COOL state:

- If the current temperature is above the setpoint, the blue LED fades in and out to indicate active cooling.

- If the temperature is below the setpoint, the blue LED remains solid.

This logic ensures that the system visually indicates when heating or cooling is actively required.

Cloud Connectivity

Although this thermostat communicates locally through UART, the Raspberry Pi itself is capable of connecting to cloud services via Wi-Fi or Ethernet. The serial data being transmitted could be redirected to a cloud service such as AWS IoT or Azure IoT Hub. Because the Raspberry Pi runs a Linux operating system, it supports networking stacks and secure communication protocols like MQTT and HTTPS. This makes the thermostat architecture scalable for real-world IoT deployment.

Device Capabilities and Architecture

The Raspberry Pi supports this application because of its ARM-based processor, Linux operating system, multitasking capability, and hardware abstraction libraries. The system uses multithreading to manage the LCD display separately from the main loop, ensuring that the user interface remains responsive. The use of interrupts for button detection improves efficiency compared to continuous polling.

The modular design of the TemperatureMachine class improves readability and maintainability. Each function has a clear responsibility, and state transitions are handled explicitly through the statemachine library. This demonstrates best practices in structured programming and object-oriented design.

Conclusion

Overall, this project successfully integrates multiple hardware peripherals into a functional thermostat system. It demonstrates GPIO control, I2C sensor communication, UART serial transmission, interrupt handling, and state machine implementation. The final system meets all specified requirements and operates reliably. This project strengthened my understanding of embedded systems architecture and real-world device integration.

References (APA Style)

Raspberry Pi Foundation. (n.d.). Raspberry Pi Documentation. https://www.raspberrypi.com/documentation/

Adafruit Industries. (n.d.). AHTx0 Temperature & Humidity Sensor Library. https://learn.adafruit.com/

Python Software Foundation. (n.d.). gpiozero Documentation. https://gpiozero.readthedocs.io/